# S32K389 Watchdog Binary Test

## Purpose

The test binary Eth_InternalLoopback_S32K389_WatchdogMulticoreFault.elf is a deliberately faulty S32K389 guest image. It verifies that the QEMU SWT watchdog detects loss of watchdog servicing and performs the configured reset or pause action without using the manual swt_trigger command.

The S32K389 model creates four Cortex-M7 cores. All cores execute the same ELF. SWT0 is shared, so one timeout event controls the board-level action.

## Binary workflow

- QEMU loads the ELF through the reset vector.

- Each core enters main() and atomically claims a diagnostic slot from shared SRAM.

- Each core records that it reached the test.

- The core assigned slot 0 unlocks and configures the shared SWT0.

- SWT0 is configured for approximately one second.

- Every core stops servicing the watchdog and enters a deterministic fault state:

  - slot 0: CPU-bound non-servicing loop;

  - slot 1: undefined instruction (UDF #0);

  - slot 2: second undefined-instruction path (UDF #1);

  - slot 3: non-servicing corrupted-state-style loop.

- SWT0 expires and QEMU performs the selected watchdog action.

The source is watchdog_multicore_fault_test.c.

## Watchdog configuration

SWT0 base address:

0x40270000

Registers used:

| Register | Address | Use |
| --- | --- | --- |
| CR | 0x40270000 | Enable watchdog with WEN = 1 |
| TO | 0x40270008 | Set timeout |
| SR | 0x40270010 | Unlock SWT |

The binary writes:

SWT0_SR = 0xC520; SWT0_SR = 0xD928; SWT0_TO = 32768; SWT0_CR = 1;

The QEMU model uses a 32,768 Hz SWT clock:

32768 counts / 32768 Hz = approximately 1 second

CR[ITR] remains clear, so the first timeout performs the configured QEMU watchdog action. The binary never writes the SWT service sequence 0xA602, 0xB480.

## Diagnostic SRAM

The binary writes debug state to shared SRAM starting at 0x2040f000:

| Offset | Meaning |
| --- | --- |
| 0x00 , 0x08 , 0x10 , 0x18 | Per-core phase |
| 0x04 , 0x0c , 0x14 , 0x1c | Per-core heartbeat |
| 0x20 | Atomic slot counter |
| 0x28 | SWT-arm status |

Inspect these values before a timeout with:

xp/12xw 0x2040f000

The heartbeat changes while a core is executing. Phase values identify the state reached by each core. Use pause mode for inspection because reset mode can immediately start another boot and overwrite the markers.

## Build locations

QEMU executable:

build/qemu-system-arm

Build QEMU:

ninja -C build qemu-system-arm

Watchdog ELF:

ELF/s32k389/Eth_InternalLoopback_S32K389_WatchdogMulticoreFault.elf

The ELF is an ARM Cortex-M7 ELF32 image and is built separately from QEMU using the existing S32DS-generated compiler/linker configuration.

## Automated tests

From the repository root:

scripts/s32k389-watchdog-test.sh reset

Expected result:

VM status: running watchdog reset test passed

The watchdog reset restarts the guest, so QEMU remains running.

Test pause behavior:

MONITOR_PORT=5600 scripts/s32k389-watchdog-test.sh pause

Expected result:

VM status: paused (watchdog) watchdog pause test passed

The script uses the multicore ELF by default. Override values when needed:

QEMU_BIN=/path/to/qemu-system-arm \ S32K389_WATCHDOG_ELF=/path/to/test.elf \ MONITOR_PORT=5600 \ RUN_SECONDS=3 \ scripts/s32k389-watchdog-test.sh reset

The script writes the QEMU log to s32k389-watchdog-test.log and verifies that it contains:

s32k3_swt[0]: watchdog timeout, performing configured watchdog action

## Manual QEMU run

build/qemu-system-arm \   -M s32k389 \   -kernel ELF/s32k389/Eth_InternalLoopback_S32K389_WatchdogMulticoreFault.elf \   -watchdog-action pause \   -d int,guest_errors \   -D /tmp/s32k389-qemu.log \   -msg timestamp=on \   -display none \   -monitor telnet:127.0.0.1:5555,server=on,wait=off

Connect to the monitor:

telnet 127.0.0.1 5555

Useful monitor commands:

info status info registers xp/12xw 0x2040f000 xp/8xw 0x40270000

The watchdog action can be selected at runtime:

watchdog_action pause watchdog_action reset

watchdog_action selects behavior; it does not trigger the timer. The automatic firmware test does not require swt_trigger. Use cont to resume after a watchdog pause.

Inspect exception and watchdog logging from the host shell, not the QEMU monitor:

grep -E 'HardFault|UsageFault|s32k3_swt|watchdog' /tmp/s32k389-qemu.log