# S32K389 NVIC Technical Report

## Result

The S32K389 board uses QEMU's generic ARMv7-M NVIC correctly at the integration level. Source inspection validates controller creation, CPU association, PPB mapping, peripheral IRQ hookup, and interrupt state management. Runtime ISR execution remains unverified because this worktree does not contain build/qemu-system-arm.

## Architecture

hw/arm/armv7m.c creates and realizes armv7m_nvic below ARMV7MState, links it to the Cortex-M CPU, connects its output to ARM_CPU_IRQ, and maps its system registers at 0xe000e000. ARMV7MState exposes the NVIC input GPIOs to the board.

The S32K389 configuration is:

- CPU: Cortex-M7

- External IRQs: 240

- Internal exception vectors: 16

- Total NVIC vectors: 256

- Implemented priority bits: 4

- Initial vector/reset address: 0x00402000

External NVIC input n represents exception 16 + n.

## Verified NVIC behavior

hw/intc/armv7m_nvic.c implements:

- enable/disable and pending/clear-pending registers

- active-state reads

- external and system-handler priorities

- priority arbitration and nesting

- exception acknowledge and completion

- SysTick and NMI inputs

- masking and privilege checks

- fault escalation and level-sensitive pending behavior

Reset initializes architectural exception priorities and state. Register writes recompute interrupt arbitration.

## Board IRQ wiring

The board connects IRQ outputs for LPUART3, FlexIO, 12 FlexCAN, 6 LPSPI, 2 LPI2C, 4 SWT, 3 ADC, 3 eMIOS, 32 eDMA channels, 2 GMAC, QuadSPI, and 2 SAI devices through sysbus_connect_irq() and qdev_get_gpio_in().

Verified or source-documented mappings:

- LPUART3

- FlexIO

- FlexCAN

- LPSPI

Unverified placeholder mappings in hw/arm/s32k389.h:

- LPI2C, SWT, ADC, eMIOS, eDMA

- GMAC, QuadSPI, SAI

Known fidelity limits:

- eMIOS is reduced to one OR'd IRQ per instance.

- Only core 0 receives modeled peripheral IRQs; cores 1–3 have independent NVICs but no peripheral routing.

## Validation status

### Passed by source inspection

- NVIC/CPU realization and association

- PPB register mapping

- CPU interrupt output connection

- vector-table configuration

- peripheral-to-NVIC GPIO wiring pattern

- enable, pending, active, priority, masking, arbitration, acknowledge, and completion paths