S32K389 Simulator vs. Real Hardware

Executive summary — what our QEMU-based simulator can and cannot tell us before we touch real chips

Prepared for management review · 2026-07-30

# The bottom line

The simulator is a lightweight stand-in for one part of the chip, not a full copy of it. It's reliable for early software development on two communication features (serial console and vehicle networking/CAN). It cannot yet be used to validate safety behavior, multi-processor coordination, or most of the chip's other capabilities (networking, sensors, storage, audio) — those still require real hardware.

# Why this matters

The real S32K389 is an automotive-grade chip built for safety-critical systems: it has four processing cores (one pair dedicated to cross-checking each other for safety), and dozens of specialized features for networking, sensing, and data handling. Building and testing firmware on real chips is slow and requires physical hardware. A simulator lets engineers write and test code on a laptop before hardware is available or without needing a physical board for every developer. The question for planning purposes is: how much of the real chip does our simulator actually stand in for, and where does the team still need real hardware?

# Key structural differences

| Area | Real chip | Simulator today | Status |
| --- | --- | --- | --- |
| Processing power | 4 processing cores, including a dedicated pair that cross-check each other for safety | 1 core, no cross-checking | Major gap |
| Safety systems | Built-in error detection and correction, secure boot, watchdog protection | None of this is simulated | Major gap |
| Vehicle networking (CAN) | 12 channels | All 12 channels simulated | Fully covered |
| Basic serial communication | 16 channels | 1 channel simulated | Partial |
| Networking, sensors, storage, audio | Ethernet, analog sensing, high-speed data transfer, audio interfaces | Not simulated | Not covered |
| Power and clock management | Fully configurable, changes at runtime | Fixed/simplified, just enough to let software boot | Simplified |
| Memory (storage and working memory) | Fixed sizes and layout | Matches the real chip | Fully covered |

# What this means for the program

- Good fit today: early firmware development and testing for features built on serial communication and vehicle networking (CAN). This is where the team can move fast without waiting on hardware.

- Not a substitute for hardware: any safety certification work, testing of the multi-core/safety architecture, or features relying on networking, sensors, or storage must still be validated on real evaluation boards.

- Risk to watch: code that looks like it's working in the simulator may behave differently on real hardware if it depends on capabilities the simulator doesn't model (timing behavior, multi-core interaction, or any of the "not covered" items above). Passing in simulation should not be read as passing on real hardware for those areas.

# Recommendation

Continue using the simulator to accelerate early-stage firmware development for CAN- and serial-based features, since that is the best-matched and most reliable part of the model. Plan real-hardware time into the schedule for anything touching safety behavior, multi-core coordination, or the peripherals the simulator doesn't cover — treat those as required hardware milestones, not simulator milestones. If broader simulator coverage would materially de-risk the schedule, that is an investment decision we can size separately.