QEMU S32K389 Model vs. the Real NXP S32K389

Architectural comparison — based on hw/arm/s32k389.c and hw/arm/s32k389.h in this repository, checked against the NXP S32K3xx datasheet, memory map, and interrupt map.

Prepared for Mahesh · 2026-07-30

# 1. Scope

The project's QEMU tree already includes a dedicated machine model for the S32K389 (hw/arm/s32k389.c / .h), separate from the S32K344 and S32K388 models in the same directory. This document compares that model's design choices against the real S32K389 silicon, as described in NXP's S32K3 datasheet and block diagram. The goal is to make clear what the emulator faithfully represents, what it simplifies, and what it omits outright.

# 2. Processor cores

| Aspect | QEMU model | Real S32K389 |
| --- | --- | --- |
| Core count | 1 (default_cpus = min_cpus = max_cpus = 1) | 4 Arm Cortex-M7 cores: one safety lockstep core plus three additional M7 cores, all up to 320 MHz |
| Lockstep | Not modeled — no compare logic, no lockstep fault injection | Dedicated lockstep core pair for ASIL D safety monitoring |
| Security core | Not modeled | Separate HSE (Hardware Security Engine) core for crypto/secure boot, isolated from the application cores |
| MPU | mpu-ns-regions and mpu-s-regions both set to 0 (effectively disabled) | Configurable MPU regions per core, part of the ASIL B/D safety case |

The single-core choice is the most consequential simplification in the model: any firmware or RTOS behavior that depends on multi-core boot sequencing, inter-core communication, or lockstep comparison cannot be exercised in QEMU today.

# 3. Clocking and mode/reset control

| Aspect | QEMU model | Real S32K389 |
| --- | --- | --- |
| System clock | Fixed virtual Clock object hard-set to 320 MHz at boot | Generated from FIRC/FXOSC and PLLs through MC_CGM, configurable and re-programmable at runtime |
| FIRC / FXOSC / MC_CGM | Collapsed into one generic read/write register stub (s32k3_clkgen_stub) covering 0x402D0000-0x402DBFFF; not a functional model | Three distinct clock-generation peripherals with real dividers, mux trees, and lock/ready status |
| MC_ME (Mode Entry Module) | Modeled at its correct address (0x402DC000, per manual §46.7.1) but only 3 offsets are handled, each returning a fixed canned value (e.g. clock-ready bit always set) | Full mode-entry state machine arbitrating power/clock partitions (RUN0-3, STANDBY, RESET) across all cores and peripherals |

The in-code comments in s32k389.c are explicit that these stand-ins exist only to satisfy firmware polling loops during boot (e.g. "clock ready" checks) and are not behaviorally accurate clock models.

# 4. Memory map

Memory sizing and layout are one of the more carefully matched areas of the model — the header comments cite verification against NXP's S32K3xx memory-map spreadsheet.

| Region | QEMU model | Real S32K389 | Match |
| --- | --- | --- | --- |
| ITCM | 64 KB @ 0x0000_0000 | 64 KB | Match |
| DTCM | 124 KB + 4 KB stack carve-out @ 0x2000_0000 | 128 KB | Match |
| Program flash | 12 MB total across 8 blocks (2×1MB + 2×2MB on "PFC1", 2×2MB + 2×1MB on "PFC0"), contiguous 0x0040_0000-0x00FF_FFFF | 12 MB across two program-flash controllers (PFC0 + PFC1), per datasheet Figure 16 "S32K389: ASIL D 12MB MCU" | Match |
| Data flash | 256 KB @ 0x1000_0000 | 256 KB (vs. 128 KB on the smaller S32K388) | Match |
| SRAM | 1920 KB across 4 banks: standby (64KB) + SRAM0 (448KB) + SRAM1 (512KB) + SRAM2 (512KB) + SRAM3 (384KB) | 1920 KB across 4 banks (512+512+512+384); S32K388 by comparison has only 3 banks / 768 KB | Match |
| ECC on memories | Not modeled — no error injection or fault reporting | ECC on SRAM and flash, part of the ASIL B/D safety features | Gap |

# 5. Peripherals

This is where the model diverges most from the real device. The real S32K389 is a large automotive gateway/domain-controller MCU; the QEMU model implements a small, hand-picked subset sufficient to boot firmware and exercise a couple of communication buses.

| Peripheral | QEMU model | Real S32K389 |
| --- | --- | --- |
| LPUART | 1 of 16 wired up and functional (instance 3, used as console); remaining 15 addresses fall into a generic "unimplemented device" catch-all that just avoids bus faults | 16 × LPUART/LIN |
| FlexIO | 1 instance modeled, used only to loop back to the console LPUART for a board-level UART example | 1 × FlexIO, general-purpose (not limited to UART loopback) |
| FlexCAN | All 12 instances implemented with correct base addresses and per-instance IRQs (cross-checked against the interrupt map) | 12 × CAN-FD — full match, the best-covered peripheral in the model |
| LPSPI | Declared in the device struct and header (6 instances, base addresses, IRQ numbers) but never instantiated in s32k389_init() — dead configuration, not actually present on the emulated bus | 6 × SPI |
| Ethernet | Not modeled | 2 × 1 Gbit/s Ethernet (TSN-capable) |
| ADC | Not modeled | 3 × 24-channel, 12-bit ADC |
| eDMA | Not modeled | 32-channel eDMA |
| I2C | Not modeled | 2 × I2C |
| SAI (I2S) | Not modeled | 2 × SAI |
| QuadSPI / uSDHC | Not modeled | 1 × QuadSPI, 1 × uSDHC (SDIO) |
| eMIOS timers / GTM | Not modeled (IRQ range formerly mis-attributed to LPSPI, since corrected in the header comments) | eMIOS timer channels, Generic Timer Module |
| Everything else in the peripheral space | 0x4000_0000, 16 MiB range registered as a single create_unimplemented_device() catch-all — reads/writes are logged and swallowed, not modeled | Populated by the rest of the peripheral set (I/O muxing, watchdogs, CRC, comparator, etc.) |

# 6. Interrupt controller

Both the model and the real part use an Arm NVIC with 240 IRQ lines (num-irq = 240, matching the S32K3 interrupt map's line count). The difference is in how many of those lines actually connect to something: in QEMU only the console LPUART, the FlexIO instance, and the 12 FlexCAN instances drive real IRQs into the NVIC. Every other IRQ source that exists on real silicon (ADC, eDMA, GTM, Ethernet, eMIOS, etc.) has no emulated peripheral behind it and therefore never fires in the emulator.

Notably, an earlier revision of this model had the LPSPI IRQ numbers wrong (69-74, which actually belong to eMIOS1); the header now documents the correction (165-170) even though LPSPI itself still isn't wired into the machine init function.

# 7. Safety and security features

| Feature | QEMU model | Real S32K389 |
| --- | --- | --- |
| Lockstep core comparison | Absent | Present — one dedicated lockstep M7 pair |
| ECC on SRAM/flash | Absent | Present |
| MPU | Configured with 0 regions (disabled) | Present, configurable |
| CRC peripheral | Absent | Present |
| Watchdogs | Absent (no watchdog device instantiated) | Present |
| HSE (crypto / secure boot) | Absent | Present, dedicated core |

In short: the emulator is a functional, single-core, non-safety-rated stand-in. It is well suited to booting and testing application-level firmware logic against UART and CAN, but it cannot be used to validate anything in the ASIL B/D safety case.

# 8. Summary

The QEMU S32K389 model prioritizes getting firmware to boot and exercising two communication peripherals (LPUART console, FlexCAN) with addresses and interrupts verified against NXP's own memory-map and interrupt-map spreadsheets. Memory sizing is also carefully matched to the datasheet. Everything past that — the extra three application cores, the lockstep pair, the HSE security core, Ethernet, ADC, eDMA, I2C, SAI, QuadSPI/uSDHC, eMIOS/GTM, and all safety mechanisms (ECC, MPU, CRC, watchdogs) — is either a passive register stub that avoids bus faults or entirely absent. LPSPI sits in between: it's declared throughout the code as if implemented, but isn't actually instantiated, which is worth fixing or removing to avoid confusion.

# 9. Practical implications

- Safe to rely on: LPUART3 console, all 12 FlexCAN instances (addresses/IRQs cross-checked), core memory map and sizes, and basic Cortex-M7 boot/vector-table behavior.

- Will silently no-op: any access to Ethernet, ADC, eDMA, I2C, SAI, QuadSPI/uSDHC, eMIOS/GTM, or LPSPI registers — these fall into the generic unimplemented-device catch-all or, for LPSPI, aren't wired to the bus at all.

- Will not exercise: multi-core startup/IPC, lockstep fault handling, HSE-based secure boot or crypto, MPU-based fault isolation, or real clock-tree reprogramming (FIRC/FXOSC/PLL/MC_CGM) — the clock-generation block is a flat register stub, and MC_ME only returns a fixed "clock ready" status rather than running the real mode-entry state machine.

- If a firmware image touches any of the absent peripherals or depends on multi-core coordination, that logic needs a real S32K389 board (or a more complete model) to validate — QEMU here is a single-core, CAN/UART-focused subset, not a full-chip emulator.